ERCOUPE

415-C

Operators

Handbook

FOR REFERENCE ONLY

Table of Contents

Critical Data	3

Weight Data	3

V Speeds	3

Preflight Inspection	4

Preflight Checklist	5

Flight Checklist	6

Shut Down Checklist	7

Weight & Balance Data	8

Emergency Checklists	9

Engine Failure During Takeoff	9

Engine Failure Immediately after takeoff	9

Engine Failure During Flight	9

Landing without Engine Power	10

Precautionary Landing with Power	10

Fire During Start on Ground	11

Engine Fire in Flight	11

Wing Fire in Flight	12

Cabin Fire in Flight	12

Electrical Fire in Flight	12

Ammeter Excessive rate of Charge or Discharge	13

Inadvertent Flight into IFR Conditions	13

Ditching	13

Inadvertent Icing	14

Engine Data	15

Engine Specifications	15

Engine Operating Limits	16

Sea Level Performance Curve	17

Altitude Performance Chart	18

Reference Section	19

METAR Reports	19

VFR Cruising Altitudes	19

VFR Weather Minimums	19

Koch Chart	20

Temperature Conversion	20

Pressure Altitude	21

Distance - Time	21

Crosswind Calculator	21

CRITICAL DATA

MAXIMUM TAKE OFF WEIGHT	1320 LBS

MAXIMUM BAGGAGE WEIGHT	40 LBS

Critical Speeds

Stall (VSO)	48 MPH

Maneuvering Speed (VA)	108 MPH

TAKEOFF & CLIMB SPEEDS

VR (Rotate)	60 MPH

VX (Best Angle of Climb)	69 MPH

VY (Best Rate of Climb)	75 MPH

OPERATING SPEEDS

Never Exceed Speed (VNE)	144 MPH

Max Structural Cruise Speed (VNO)	114 MPH

Normal Operation Range	50-114 MPH

Caution Range	114-144 MPH

Max Demonstrated Crosswind	21.5 KTS

DESCENT SPEEDS

Final	75-80 MPH

VGLIDE	75 mph

FUEL CAPACITY

Header Tank	6 gal

Wing Tank	9 gal each

PRE-FLIGHT INSPECTION

Weather / Performance Data	CHECK

PRE-FLIGHT INSPECTION

Money for Fuel (X-Country)	CHECK

Keys	ON DASHBOARD

EMPENAGE

Skins & Rivets	UNDAMAGED

Vertical Fins	SECURE

Rudder – Movement/Security (Outboard movement only)	CHECK

Elevator – Movement/Security	CHECK

White Navigation Light	CHECK

Tie Down	REMOVED

RIGHT WING

Right Aileron Security	CHECK

Right Navigation Light	CHECK

General Condition of Fabric	CHECK

Right Brakes and Strut	CHECK

Right Tire Pressure (17 psi)	CHECK

Tie Down	REMOVED

Right Wing Tank (Cap vent must be forward)	CHECK & SUMP

NOSE

Header Tank (Cap vent must be forward)	CHECK & SUMP

Oil Quantity (Max = 4.5 qt; Min = 2 qt)	CHECK

Right Cowling	CHECK & SECURE

Nose Tire (20 psi)	CHECK

Propeller & Spinner	CHECK

Strut	CHECK

Vents & Bracket Air Filter	CHECK

Gascolator	CHECK & SUMP

Left Cowling	CHECK & SECURE

LEFT WING

Left Wing Tank (Vent in cap must be forward)	CHECK & SUMP

Left Brakes and Strut	CHECK

Left Tire Pressure (17 psi)	CHECK

Pitot Cover	REMOVED

Tie Down	REMOVED

Left Navigation Light	CHECK

Left Aileron Security	CHECK

General Condition of Fabric	CHECK

One Last 360° Walkaround	COMPLETE

PRE-FLIGHT CHECKLIST

PRE-FLIGHT CHECKLIST

BEFORE START

Pre-flight Inspection	COMPLETE

Airworthiness Certificate	CHECK

Registration	CHECK

Weight & Balance	CHECK

Passenger Brief	COMPLETE

Seat Belts	SECURE

Baggage Compartment 	SECURE

Fuel Cross Feed	BOTH

Fuel Valve	ON

Radios, Intercom, & Panel Lights	OFF

Headsets	CHECK

Circuit Breakers	CHECK

Brakes	SET

Strobe Light	ON

Carb Heat	OFF / COLD

Key		IN IGNITION SWITCH

MASTER	ON

COLD ENGINE

Primer	2 – 6 Strokes

Primer	LOCKED

Throttle	OPEN ¼ INCH

ENGINE START

Magnetos	ON (BOTH)

Propeller Area	CLEAR

Starter	PULL

Oil Pressure	CHECK (10 PSI MIN)

Transponder	STBY

Intercom	ON

Navigation Lights	ON

TAXI

ATIS	LISTEN & RECORD

Traffic	LOOK & LISTEN

Taxi Clearance	OBTAINED

Brakes	CHECK

Compass	CHECK

FLIGHT CHECKLIST

FLIGHT CHECKLIST

BEFORE TAKEOFF

Brake	ON

Controls (Elevator Only)	FREE

Throttle	1700 RPM

Ignition Check (Max drop 150 RPM, 75 RPM Difference)	CHECK

Carb Heat (Verify drop and stable)	CHECK

Oil Pressure	30 PSI (MIN)

Oil Temperature	100° (MIN)

Throttle	1000 RPM

Altimeter and Radio	SET

Transponder	ALT

Elevator Trim	SET

Traffic	CHECK

TAKEOFF

Carb Heat	COLD (IN)

Throttle	FULL OPEN

Elevator	Lift Nose Wheel at 60 MPH

Climb	70-80 MPH

CRUISE

Throttle	SET (2120 – 2300 RPM)

Oil Pressure	30 -40 PSI

Elevator	TRIM

LANDING

Carb Heat	FULL HEAT (OUT) before closing throttle

Best Glide	75 MPH

Approach	75 MPH (power) - 80 MPH (no power)

GO AROUND

Carb Heat	COLD (IN)

Throttle	FULL

Elevator	Lift Nose Wheel at 60 MPH

Climb	70-80 MPH

AFTER LANDING

Carb Heat	COLD (IN)

SHUT DOWN CHECKLIST

SHUT DOWN CHECKLIST

SHUT DOWN

Lights	OFF

Transponder	OFF

Radios	CHECK 121.5

Radios	OFF

Header Tank Fuel Valve	OFF

Magnetos	OFF

Master Switch	OFF

Keys	ON DASH

SECURE AIRPLANE

Pitot Cover	INSTALLED

Tie Downs	SECURE

Skin	WIPE DOWN

Nose Gear Strut	WIPE w/OIL

Windows	CLOSED & LOCKED

Cover	INSTALLED

Phone Numbers

Flight Service	1-800-WX-BRIEF (1-800-992-7433)

Weight & Balance Data

WEIGHT AND BALANCE DATA

| Item |  | Weight | Arm | Moment |
| --- | --- | --- | --- | --- |
| Empty Airplane |  | 888 | 26.376 | 23422 |
| Equipment List |  |  |  |  |
| ELT |  |  |  |  |
| Radio (icom IA200) |  |  |  |  |
| Xponder |  |  |  |  |
| Blind Encoder |  |  |  |  |
| Day to Day Operations | Qty | Fill in white spaces & calculate |  |  |
| Fuel -Header tank (gal) |  |  | 7 |  |
| Fuel -Wing tanks (gal) |  |  | 25 |  |
| Usable Oil (4.5 quarts) |  | 8.3 | -14 | -103.6 |
| Pilot |  |  | 37 |  |
| Passenger |  |  | 37 |  |
| Baggage |  |  | 62 |  |
| Totals |  |  |  |  |
| Center of Gravity (CG) |  |  |  |  |
| CG Range | 26.4” to 30.3” |  |  |  |
| Formulas: Weight x Arm= Moment; ∑Moments/ ∑Weight = CG |  |  |  |  |
| Power Setting |  | 65% | 75% | 100% |
| Fuel Rate | gph | 3.6 | 4.1 | 5.5 |
| Reserve (30 min) | gal | 1.8 | 2.1 | 2.8 |
| Tot. Flying Time w/res. | hrs | 2.86 | 2.41 | 1.68 |
| Flying Time NO res. | hrs | 3.36 | 2.91 | 2.18 |
| Indicated Airspeed | mph | 96.0 | 108.0 | 120 |
| Range w/reserves | (mi) | 274.2 | 260.2 | 201.8 |
| Range NO reserves | (mi) | 322.2 | 314.2 | 261.8 |
| Operating Limits |  |  | Difference |  |
| 415-C (max Gross) | 1320 lbs |  |  |  |

### EMERGENCY CHECKLIST

Best Glide Speed	78 MPH

# ENGINE FAILURE DURING TAKEOFF RUN

- Throttle	IDLE

- Brakes	APPLY

- Ignition Switch 	OFF

- Master Switch	OFF

# ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

- Airspeed	78 MPH

- Main Fuel Selector Valve	OFF (FWD)

- Fuel Transfer Valve	OFF

- Ignition Switch 	OFF

- Master Switch	OFF

- Canopy…	DO NOT OPEN

# ENGINE FAILURE DURING FLIGHT

- Airspeed	78 MPH

- Carb Heat	FULL ON (Pull)

- Main Fuel Selector Valve	VERIFY ON

- Fuel Transfer Valve	VERIFY ON

- Ignition Switch 	BOTH

- Primer	IN & LOCKED

- Starter Button	PULL (If Prop is Stopped)

- Radio	121.5 - MAYDAY & POSITION

- Transponder	SQUAWK 7700

- ELT	Activate

### EMERGENCY CHECKLIST

# EMERGENCY LANDING WITHOUT ENGINE POWER

- Airspeed	78 MPH

- Main Fuel Selector Valve	OFF (FWD)

- Fuel Transfer Valve	OFF

- Ignition Switch 	OFF

- Master Switch	OFF

- Seat Belts	SECURE

- Passenger 	BRIEF (No Feet on Control Mast)

- Canopy…	DO NOT OPEN

- Touchdown	SLIGHTLY TAIL LOW

- Brakes	APPLY HEAVY

### PRECAUTIONARY LANDING WITH ENGINE POWER

- Airspeed	80 MPH

- Seat Belts	CHECK

- Passenger 	BRIEF (No Feet on Control Mast)

- Selected Field

FLY OVER, INSPECT, CK WIND DIR.

- Radio	121.5 - MAYDAY & POSITION

- Master Switch	OFF

- Canopy	UNLATCH - DO NOT OPEN

- Carb Heat	FULL ON

- Approach Speed	78 MPH UNTIL SHORT FINAL

- Touchdown	SLIGHTLY TAIL LOW

- Ignition Switch	OFF

- Brakes	APPLY HEAVILY

### EMERGENCY CHECKLIST

FIRE DURING START ON GROUND

- Cranking	CONTINUE to get a start

If Engine Starts:

- Power	1700 RPM for 2 MINUTES

- Engine	SHUTDOWN AND INSPECT

If Engine Won't Start:

- Cranking	STOP Cranking

- Passenger	EXIT to REAR of Airplane

- Throttle	FULL OPEN

- Master Switch	OFF

- Ignition Switch	OFF

- Main Fuel Valve	OFF (Handle to Right)

- Fuel Transfer Valve	OFF

- Fire	EXTINGUISH with Extinguisher

- Fire Damage	INSPECT

# ENGINE FIRE IN FLIGHT

- Main Fuel Valve	OFF

- Fuel Transfer Valve	OFF

- Master Switch	OFF

- Cabin Heat Knob	OFF (PUSH IN)

- Cabin Air Knob	OFF (PUSH IN)

- Airspeed	115 MPH

If not extinguished	Increase glide speed

- Engine	DO NOT ATTEMPT RESTART

- Forced Landing	EXECUTE Emergency Landing

Without Power

### EMERGENCY CHECKLIST

# WING FIRE IN FLIGHT

- Navigation Light Switch	OFF

- Airplane	SIDESLIP to keep flames away

from fuel tank and cabin

- Flight	LAND as soon as practical

# CABIN FIRE IN FLIGHT

- Master Switch	OFF

- All Vents, Cabin Heat, Air	OFF / CLOSED

- Fire Extinguisher	ACTIVATE

- Cabin	VENTILATE AFTER fire is out.

- Flight	LAND as soon as practical.

# ELECTRICAL FIRE IN FLIGHT

- Master Switch	OFF

- All Switches Except Ignition	OFF

- All Vents, Cabin Heat, Air	OFF / CLOSED

- Fire Extinguisher	ACTIVATE

- Cabin	VENTILATE as required to clear

extinguishing agent

- If Fire is out and electrical power is REQUIRED for continuance of flight:

- Radio Switches	OFF

- Master Switch	ON

- Circuit Breakers	CHECK for faulty circuit

DO NOT RESET

- Radio/Electrical Switches, IF REQUIRED:	ON

One at a time with delay after each, until short circuit is localized

- Vents/Cabin Air/Heat	OPEN when it is

ascertained that fire is completely extinguished

- Flight	LAND as soon as practical

### EMERGENCY CHECKLIST

# AMMETER SHOWS EXCESSIVE HIGH RATE OF CHARGE OR DISCHARGE

- Generator Switch	OFF

- Non-Essential Switches	OFF

- If Abnormal Condition Not Corrected:

- Master Switch	OFF

- Flight	LAND as soon as practical.

# INADVERTENT FLIGHT INTO IFR CONDITIONS

- Power	2200 RPM

- Direction	Note Heading

- Flight	Execute Standard Rate 180 turn

# DITCHING

- Radio	121.5 - MAYDAY & POSITION

- Transponder	SQUAWK 7700

- Heavy Objects	SECURE OR JETTISON

- Approach

High Winds, Heavy Seas - INTO THE WIND

Light Winds, Heavy Swells - PARALLEL TO SWELLS

- Power	300 FPM DESCENT AT 55 MPH

- If NO POWER	APPROACH AT 75 MPH

- Canopy	OPEN

- Touchdown	LEVEL ATTITUDE

At Established rate of decent

8.	Face	CUSHION at touchdown with folded coat

- Airplane	EVACUATE IMMEDIATELY

- Life Vests	INFLATE

### EMERGENCY CHECKLIST

# INADVERTENT ICING ENCOUNTER

- Airplane	TURN BACK / CHANGE ALTITUDE

to attain an OAT less conducive to icing.

- Cabin Heat	FULL ON

- Throttle	OPEN to increase engine speed and

minimize ice buildup on propeller blades.

- Carb Heat	WATCH for loss of power and

APPLY as required

- Landing	LAND as soon as practical

If rapid ice build-up, land off airport

- Stall Speed	Expect Higher Stall Speed

- Canopy	OPEN if required for visibility

for landing approach

SCRAPE windshield, if practical

- Landing	Perform using forward slip

if required for visibility.

- Approach	85 to 90 MPH depending

on amount of ice accumulation

- Land	LEVEL ATTITUDE

Expect higher stall speed.

Engine Specifications for C85

Type Certificate No.	233

Bore (inches)	4 - 1/16

Stroke (inches)	3 -5/8

Comp. Ratio	6.3:1

Piston Disp. (cu. In.)	188

Rated Power (sea Level)	85

Rated RPM	2575

Max Allowed Press. (in Hg.) at rated RPM at Sea Level	29

Recommended Crusing RPM	2400

Max. Rec. Manifold Pressure (in Hg.) for cruising at SL	24.5

Minimum Fuel Grade	80/87

Oil Pressure at Cruising (psi)	30-35

Min. Idling Oil Pressure (psi)	10

Minimum Oil Temperature - F°	75

Maximum Oil Temerature - F°	225

Max. Cylinder Head Temperature - F°	540

Approx. Fuel consumption in Gals./Hour Cruising	5.4

Approx Max. Desirable Oil Consumption in Qts/hr.	.4

Oil Sump Capacity in Qts	4.5

Tappet Clearance:

Operating	0

Lifters Deflated	.030-.110

Ignition Timing BTC

Right	28°

Left	30°

Engine Operating Limits C-85

Model C75 & C85 – Sea Level Performance Curve

Model C75 & C85 – Altitude Performance Curve

| METAR Reports Item | Description | Sample |
| --- | --- | --- |
| Message Type | METAR (hourly), TAF | METAR |
| Location |  | KCRQ |
| Issuance Time | DDTTTT (Zulu) | 091955Z |
| Observation | CORrected or AUTOmatic | COR |
| Wind | directionor VAR, speed, Gust | 22015G25KT |
| Visibility | Statute Miles | 4SM |
| Runway Vis Range | R, runway / range | R28L/2600FT |
| Significant Weather | See Chart | TSRA |
| Cloud Cover | Amount, height and Type | OVC010CB |
| Temp/Dewpoint | M for below zero | 18/16 |
| Altemeter Setting | A for inches and Hundredths | A2992 |
| Remarks | Sea-level press. In tenths hPa | RMK SLP045 |
|  | Temp/dewpoint in tenths iC | T01820159 |

VFR CRUISING ALTITUDES

Codes

BC	Patches

BL	Blowing

BR	Mist

CB	Cumulonimbus

DR	Low Drifting

DS	Dust Storm

DU	Dust

DZ	Drizzle

+FX	Tornado

FC	Funnel Cloud

FG	Fog

FU	Smoke

FZ	Freezing

GR	Hail

GS	Hail/Snow Pellets

HZ	Haze

IC	Ice Crystals

MI	Shallow

PE	Ice Pellets

PO	Dust/Sand Whirls

PR	Partial

PY	Spray

RA	Rain

-RA	Light Rain

SA	Sand

SG	Snow Grains

SH	Showers

SN	Snow

+SN	Heavy Snow

SQ	Squall

SS	Sand Storm

TCU	Towering CB

TS	Thunderstorms

UP	Unknown Precip.

VA	Volcanic Ash

VFR WEATHER MINIMUMS

| Class | Altitude | Vis (sm) | Cloud Distance |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | above | below | lateral |
| A | All | X | No VFR Allowed |  |  |
| B | All | 3 | Clear of Clouds |  |  |
| C | All | 3 | 1000 | 500 | 2000 |
| D | All | 3 | 1000 | 500 | 2000 |
| E | <10,000 MSL | 3 | 1000 | 500 | 2000 |
| E | >=10,000 MSL | 3 | 1000 | 1000 | 1 sm |
| G Day | <=1,200 AGL | 1 | Clear of Clouds |  |  |
| G Day | >1,200 AGL – 10,000 MSL | 1 | 1000 | 500 | 2000 |
| G Nite | <=1,200 AGL | 3 | 1000 | 500 | 2000 |
| G Nite | 1,200SGL – 10,000 MSL | 3 | 1000 | 500 | 2000 |
| G | >1200 AGL & 10,000 MSL | 5 | 1000 | 1000 | 1 sm |

180° to 359°

Even Thousands

MSL + 500’

0° to 179°

Odd Thousands

MSL + 500’

KOCH CHART

CARBURETOR ICE REFERENCE CHART

Light Signal	Ground	Flight

Green (steady)	Cleared for Takeoff	Cleared to Land

Green (flashing)	Cleared to Taxi	Return for Landing

Red (steady)	STOP	Give way to others

Red (flashing)	Taxi clear of runway	Airport UNSAFE

White (flashing)	Return to starting point	N/A

Alt. Red/Green	CAUTION	CAUTION

PRESSURE ALTITUDE

CROSSWIND CALCULATION

Example:

A) 60° angle between wind and nose

B) 20 knot wind speed

C) 10 knot headwind component

D) 18 knot crosswind component

Distance-Time

Distance

Spd		5	10	15	20

70	4:17	8.34	12:51	17:09

80	3:45	7:30	11:15	15:00

90	3:20	6:40	10:00	13:20

100	3:00	6:00	9:00	12:00

110	2:43	5:27	8:10	10:54

120	2:30	5:00	7:30	10:00

CONVERSIONS

DISTANCE	Multiplier

SM to Nautical Miles	.869

Nautical Miles to SM	1.610

SPEED:

Knots to MPH	1.151

MPH to Knots	1.609

TIME:

Eastern:	-5:00

Central:	-6:00

Mountain	-7:00

Pacific	-8:00

London	GMT (Z)

Avgas	6 lbs/gal

Oil	1.875 lbs/qt

Water	8.35 lbs/gal

Have a great flight!